# LM5148 Equations (Datasheet pages 36–39)

Start by entering the input voltage (VIN), output voltage (VOUT), and output current (IOUT) requirements

Use Equation 31 to calculate the required buck inductance based on a 30% ΔILO inductor ripple current at nominal input voltages.

## Equation (31): Buck inductance

L = Vout*Vinnom- VoutVinnom*Fsw*ΔILo

Use Equation 32 to calculate the peak inductor currents at maximum steady-state input voltage. Subharmonic oscillation occurs with a duty cycle greater than 50% for peak current-mode control. For design simplification, the LM5148 has an internal slope compensation ramp proportional to the switching frequency that is added to the current sense signal to damp any tendency toward subharmonic oscillation.

## Equation (32): Peak inductor current

ILOpk= Iout +Vout*Vinmax- VoutVinmax*L*Fsw2

## Equation (33): Slope compensation cross-check (as shown)

RO(sc) = Vout*RsL*Fsw

## Equation (34): Current-sense resistor

Rs =Vcsth1.25*ILpk

## Equation (35): Short-circuit peak inductor current

IL_pk(sc) = (VcsthRs) + Vin_max*tdelayL

## Equation (36): Output capacitance for load-off overshoot

Cout = L*Iout2 Vout+ΔV2- Vout2

## Equation (37): Output ripple (capacitive + ESR)

ΔVoutpp≈ sqrt ΔILo8*Fsw*Couteff2 + ΔILo*RESR2

## Equation (38): Output capacitor RMS ripple current

Icorms=ΔILosqrt12

## Equation (39): Input capacitor RMS ripple current

Icinrms= Iout*sqrtD*1-D

## Equation (40): Input capacitance (with ESR term)

Choose Cin such that ΔVin_pp meets spec; one form uses ΔVin_cap ≈ Iout*D*(1-D)/(Fsw*Cin) with ESR ripple added

## Equation (41): Frequency set resistor

FswkHz=10645*RtkΩ+ 53

## Equation (42): Feedback resistors

Vout = Vref*1 +RtopRbottom

## Equation (43): RCOMP (per datasheet procedure)

RCOMP =Vout*Rs*Gm2*π*fC*Couteff*Vref

## Equation (44): CCOMP zero placement

CCOMP =102*π*fC*RCOMP

## Equation (45): CHF pole at ESR zero

CHF =12*π*fESR*RCOMP- Cbw